Power Measurements—Lab 13

M.V. Iordache, EEGR2051 Circuits and Measurements Lab, Fall 2020, LeTourneau University

See https://mviordache.name/EEGR2051 for more information.

Name_________________________			Date__________			Section_____

EEGR 2051 – Power Measurements

Materials

- One 10 Ω resistor, one 220 Ω resistor, and one 180 Ω resistor.

- One 30 mH inductor.

- One capacitor substituter and one resistor substituter.

Procedure

- Measure the three resistors and indicate their actual value.

| Nominal Value | r=10 Ω | R1=180 Ω | R2=220 Ω |
| --- | --- | --- | --- |
| Actual Value |  |  |  |

- Set the waveform generator to a sine wave of 600 Hz and 4 V peak-to-peak.

- Verify the voltage of the generator with the oscilloscope.

- From now on, let’s denote by r, R1, and R2 the three resistors, as in the table above.

- Connect the following circuit.

- Select manually the range of the ammeter to a value between 20 mA and 500 mA.

- Note that the r=10 Ω resistor will be used to sense the current.

- Connect CH1 of the oscilloscope to the waveform generator and CH2 to the sensing resistor.

- Does the voltage of the source change when you connect the source to the circuit? Explain why.

_______________________________________________________________________________________

- Measure the phase angle difference between the source voltage and the source current. Write the answer in degrees. Δϕ= ________.

- Indicate whether the current leads or lags the voltage. _______

- The power factor is ____________________.

- Measure the rms value of the load voltage vo shown in the figure above. Vo=  _____________

- Calculate the apparent power of the load by multiplying V0 by the rms current. S= __________

- Measure the rms voltage of the inductor. VL= _______________

- The reactive power is Q= ______________

- Measure the rms voltage on either of R1 or R2. VR= ___________.

- The average power absorbed by R1 is P1= ____________.

- The average power absorbed by R2 is P2= ____________.

- The total average power is P= __________.

- Find P2+Q2. ____________

- Is the equality S= P2+Q2 verified? ________________________

- To correct the power factor, set the capacitor substituter to 100 nF and connect it as in the figure.

- Increase the value of the capacitor substituter and verify that the phase angle difference changes.

- When the power factor is corrected, the load current iS is minimum, and the phase angle difference is zero.

- Adjust the value of the capacitor substituter until the rms value of the current reaches the minimum value. The value at which the power factor is corrected should be in the range 100…3000 nF. C= __________

- The phase angle difference is Δϕ= ________.

- The following part of the experiment will demonstrate that power factor correction is load dependent, that is, changes in the load require readjusting the value of the capacitor.

- Disconnect the resistor R2 from the circuit.

- The phase angle difference is now Δϕ= ________.

- Measure the rms values of the load voltage vo and the load current is. Vo= __________	IS= ________

- The apparent power of the load is S= ___________.

- Measure the rms voltage on R1. V1= ________

- The average power absorbed by R1 is P1= __________.

- Should the apparent power S equal P1? _______________________________________________________

- Readjust the capacitor substituter until the power factor is corrected.  C= __________

- Measure again the rms voltage on R1. V1= ________

- The average power absorbed by R1 is P1= __________.

- By Ohm’s law, the rms value of the resistor current is I1= __________.

- Measure the rms value of the inductor voltage. VL= ____________.

- The reactive power of the inductor is QL= ______________.

- Measure the rms value of the capacitor current. IC= __________

- Measure the rms value of the voltage vo. Vo= ____________

- The reactive power of the capacitor is QC= ______________.

- The total reactive power of the load is Q= ____________. Is this correct? ___________________________

_______________________________________________________________________________________

- Measure the rms value of the load current is. IS= __________

- The apparent power of the load is S= ___________.

- Should S equal P1? _______________________________________________________________________

- Many pieces of modern equipment tend to absorb power from the supply in an irregular fashion, as they seek to reduce power consumption. However, this can distort the supply voltage to the point that it no longer resembles a pure sinewave. The following questions evaluate the effect of the distortion on the efficiency of a power system.

- Note that a zero-average signal vt of frequency ω can be decomposed into the sum

vt=A1cosωt+α1+A2cos2ωt+α2+A3cos3ωt+α3+…

where A1cosωt+α1 is the fundamental and Ancosnωt+αn is the harmonic of order n.

- The harmonic distortion of a signal can be calculated with the formula

Harmonic distortion=Vrms2-A1,rms2A1,rms

where Vrms is the rms value of v(t) and A1,rms=A12 is the rms value of the fundamental.

- For a pure sinusoidal signal of frequency ω, the amplitude of the harmonics is A2=A3=A4=…= _____

Moreover, the harmonic distortion is ______.

- Set the waveform generator to a triangle wave. Adjust the amplitude of the waveform generator until the rms voltage on R1 equals the value found at part ‎37. This step is important in order to ensure that the triangle wave dissipates as much power on the load as the sinusoidal signal.

- Measure the rms value of the load current is. IS= ______________.

- Using the oscilloscope, find the rms value of the first harmonic of the source voltage vS.

  - Set the time base on 50 ms/div.

  - Press the MATH button, the Operator soft key, and select FFT.

  - Set Source 1 to the channel on which you have the source voltage.

  - If lit, press twice the 1 and 2 buttons to turn off the channels 1 and 2.

  - Press the MATH button and set Span to 5 kHz and Center to 2.5 kHz.

  - Press More FFT and then Window; select Rectangle. Do not press Auto Setup; if you do, you may have to go back to step (a).

  - The largest peak on the screen is the fundamental.

  - Using cursors, measure the value of the fundamental: A1,dB=  ____ dB.

  - Convert dB to rms volts. Note that A1,dB=20log10A1,rms.

  - A1,rms= __________.

- Measure the rms value of the source voltage vS. Vs, rms= ____________

- Calculate the harmonic distortion: ______________

- Set the waveform generator to a square wave. Adjust the amplitude of the waveform generator until the rms voltage on R1 equals the value found at part ‎37.

- Measure the rms value of the load current is. IS= ______________.

- Using the oscilloscope, find the rms value of the first harmonic. A1,rms= __________.

- Measure the rms value of the source voltage vS. Vs, rms= ____________

- Calculate the harmonic distortion: ______________

- Compare the values of the load current IS found at steps ‎46, ‎54, and ‎59. To reduce the load current, is it better to use a source with high or low harmonic distortion? _________________

Maximum Power Transfer

- To enhance the efficiency of a power transmission system, it is important to ensure that most power is dissipated on the load and very little power is lost at the source. However, when working with low-power sources, such as the antenna of a receiver, we are not interested in getting the signal as efficiently as possible to the load, but in transferring as much power as possible from the source to the load. It can be proven that we have maximum power transfer when the impedance of the load Zo is related to the impedance of the source ZS by the equation Zo=ZS*. (Note that ZS* denotes the complex conjugate of ZS.) While this implies only 50% efficiency, the load gets the maximum possible amount of power from the source.

- Connect the following circuit. Using substituters, set R=100 Ω, L=30 mH, and C=100 nF.

- In this circuit, the load will consist of R and C.

- Set the waveform generator to a sinusoidal signal of 5 V peak-to-peak and 600 Hz.

- Create a LabVIEW virtual instrument that displays the average power and the graph of the instantaneous power of the load.

  - To determine instantaneous power, the instrument should measure the voltage on r, to determine the current. It should also measure the voltage on R.

  - The instrument should divide the voltage on r by r and then multiply the result by the voltage on R. The result of the multiplication is the instantaneous power. This should be averaged to find the average power.

  - The instrument should acquire data at a rate of 100,000 samples per second for 100 ms. The total number of samples in 100 ms will be _________.

- Adjust the value of the capacitor until the VI shows maximum power. This value should be between 100 nF and 2 μF. C= ____________

- Measure the rms value of the capacitor voltage. VC= _________

- Measure the rms value of the inductor voltage. VL= _________

- Are the inductor and capacitor voltages equal? Should they be equal? ______________________________

- Adjust the value of the resistor R until the VI reports maximum power. The optimal value should be between 10 Ω and 100 Ω. R= _________

- Read the output resistance of the waveform generator from its front panel. Rout= ______

- Does R equal Rout+r? Should they be equal? _________________________________________________